# Methyl ethyl ketone (MEK)

| CAS number: | 78-93-3 |
| --- | --- |
| Synonyms: | 2-Butanone, butan-2-one, MEK |
| Chemical formula: | C4H8O |
| Structural formula: | — |

Workplace exposure standard (amended)

| TWA: | 200 ppm (590 mg/m3) |
| --- | --- |
| STEL: | 300 ppm (885 mg/m3) |
| Peak limitation: | — |
| Notations: | Sk. |
| IDLH: | 3,000 ppm |
| Sampling and analysis: The recommended value is quantifiable through available sampling and analysis techniques. |  |

## Recommendation and basis for workplace exposure standard

A TWA of 200 ppm (590 mg/m3) is recommended to protect for irritant effects and adverse effects on the central nervous system (CNS) in exposed workers.

A STEL of 300 ppm (885 mg/m3) is recommended to protect for acute irritant effects in exposed workers.

## Discussion and conclusions

Methyl ethyl ketone (MEK) is used as a solvent, mainly in coatings, and often in mixtures with acetone, ethyl acetate, n-hexane, toluene or alcohols.

Critical effects from exposure include CNS effects, headache, irritation of the eyes and upper respiratory tract.

No adverse effects were reported following inhalation exposure at 200 ppm for four hours in volunteer human studies, while exposure at 300 ppm caused headaches, throat irritation and local irritation effects (DFG, 1997). Limited quality data suggests potential for irritant effects at levels of 100 ppm (ACGIH, 2001). However, the likelihood of critical CNS effects of exposure is reduced by the irritation effect of the chemical on eyes and upper respiratory tract mucosa (SCOEL, 1999). SCOEL calculated occupational exposure limit based on the identified NOAEC of 2,500 ppm in rats and data from volunteer study.

A TWA of 200 ppm by ACHIH (2001), DFG (1997) and SCOEL (1999) is recommended to protect for irritant effects in exposed workers. A STEL of 300 ppm is recommended to protect for acute exposure effects including headaches and adverse effects on peripheral nerves in exposed workers.

## Recommendation for notations

Not classified as a carcinogen according to the Globally Harmonized System of Classification and Labelling of Chemicals (GHS).

Not classified as a skin sensitiser or respiratory sensitiser according to the GHS.

A skin notation is recommended as evidence indicates rapid absorption through the skin in animals and humans increasing systemic burden.

# Appendix

### Primary sources with reports

| Source Year set Standard |  |
| --- | --- |
| SWA 1991 TWA: 150 ppm (445 mg/m3); STEL: 300 ppm (890 mg/m3) |  |
| ACGIH 2001 TLV-TWA: 200 ppm (590 mg/m3); TLV-STEL: 300 ppm (885 mg/m3) |  |
| TLV-TWA and TLV-STEL recommended to minimise the potential for irritant effects to eyes and upper respiratory tract and adverse effects on CNS. Summary of data: Human studies: - Slight nose and throat irritation at 100 ppm and mild eye irritation at 200 ppm, deemed to be a “practical limit” when working with MEK - CNS effects and peripheral neuropathy reported at 150–450 ppm, with mixtures of other organic solvents and unmonitored exposure leading to wide range excursions reported - Epidemiological study of 446 workers from dewaxing plants for cancer mortality found no increased cancer risk. Animal studies: - LD50: 6.86 mL/kg (rats, oral) - Rats exposed at 6,000 ppm (8 h/d, 7 d/wk) did not produce obvious motor impairment: - all rats died from bronchopneumonia in wk 7 - MEK in mixtures with methyl n-butyl ketone, n-hexane and 2,5-hexanedione increased development or severity of neuropathies in animals - Negative results in Salmonella assay and cultured Chinese hamster ovary cells. Potentiation may occur in other organic solvents when used in a mixture with MEK. Insufficient data to recommend skin, SEN or carcinogenicity notations. |  |
| DFG 1997 MAK: 200 ppm (600 mg/m3) |  |
| Summary of additional data: Human data: - No statistically significant differences in motor reaction, visual vigilance, dual tasks and short-term memory tests between controls and people exposed at 200 ppm for 4 h - In volunteer study, exposure at 300 ppm caused headaches, throat irritation and local irritation effects - In volunteer study, repeat exposure at 300–600 ppm caused dermatosis and numbness in fingers and arms - Poor documentation and methodology techniques discounted findings of nose and throat irritations observed at 100 ppm. Animal data: - Acute toxicity of orally administered MEK is low - NOAEL: 2,500 ppm (rats, 6 h/d, 5 d/wk for 90 d) - LD50: >8,000 mg/kg (rabbit, dermal), however rapid percutaneous absorption noted - Eye irritation observed at application of 0.1mL (rabbit). MAK retained as no significant effects reported in humans exposed at 200 ppm and irritative symptoms described at and above 300 ppm. |  |
| SCOEL 1999 TWA: 200 ppm (600 mg/m3); STEL: 300 ppm |  |
| Summary of additional data: Human data: - Predominant exposure via inhalation with rapid absorption through skin - Limited studies of systemic effects due to irritant effects on eyes and mucous membranes preventing uptake of higher doses - Central nervous depressive effects observed at concentrations >300 ppm - Most human data involve exposure to solvent mixtures and therefore inconclusive for MEK exposure alone. Animal data: - NOAEL: 2,500 ppm (rats, 6 h daily, 5 d/wk for 90 d) - Evidence MEK potentiates the neurotoxicity of methyl n-butylketone, ethyl butylketone (EBK) and n-hexane. The calculated TWA is derived from the sub-chronic rat study with NOAEL of 2,500 ppm. An UF of 10 was applied to extrapolate to chronic study, resulting in exposure level of 250 ppm. This calculated value is supported by volunteer human study of 10 workers with no adverse effects at 200 ppm exposure. Limited data exists for short duration exposures with the STEL of 300 ppm recommended to limit exposure peaks causing irritation effects retained from study conducted in 1951. |  |
| OARS/AIHA NA NA |  |
| No report. |  |
| HCOTN NA NA |  |
| No report. |  |

### Secondary source reports relied upon

NIL.

### Carcinogenicity — non-threshold based genotoxic carcinogens

| Is the chemical mutagenic? | Insufficient data |
| --- | --- |
| Is the chemical carcinogenic with a mutagenic mechanism of action? | Insufficient data |
| Insufficient data are available to determine if the chemical is a non-threshold based genotoxic carcinogen. |  |

## Notations

| Source | Notations |
| --- | --- |
| SWA | — |
| HCIS | — |
| NICNAS | — |
| EU Annex | NA |
| ECHA | — |
| ACGIH | — |
| DFG | H (skin) |
| SCOEL | Skin |
| HCOTN | NA |
| IARC | NA |
| US NIOSH | NA |

NA = not applicable (a recommendation has not been made by this Agency); — = the Agency has assessed available data for this chemical but has not recommended any notations

### Skin notation assessment

Calculation

Conclusion:. Adverse effects in human case study:; yes; 4.00. Dermal LD50 ≤1000 mg/kg:. Dermal repeat-dose NOAEL ≤200 mg/kg:. Dermal LD50/Inhalation LD50 <10:. In vivo dermal absorption rate >10%:. Estimated dermal exposure at WES >10%:. a skin notation is warranted.

### IDLH

| Is there a suitable IDLH value available? | Yes |
| --- | --- |

## Additional information

| Molecular weight: | 72.11 |
| --- | --- |
| Conversion factors at 25°C and 101.3 kPa: | 1 ppm = 2.94 mg/m3; 1 mg/m3 = 0.340 ppm |
| This chemical is used as a pesticide: | ☐ |
| This chemical is a biological product: | ☐ |
| This chemical is a by-product of a process: | ☐ |
| A biological exposure index has been recommended by these agencies: |  ACGIH  DFG ☐ SCOEL |

## Workplace exposure standard history

| Year | Standard |
| --- | --- |
| Click here to enter year |  |

## References

American Conference of Industrial Hygienists (ACGIH®) (2018) TLVs® and BEIs® with 7th Edition Documentation, CD-ROM, Single User Version. Copyright 2018. Reprinted with permission. See the TLVs® and BEIs® Guidelines section on the ACGIH website.

Deutsche Forschungsgemeinschaft (DFG) (1999) 2-Butanone – MAK value documentation.

EU Scientific Committee on Occupational Exposure Limits (SCOEL) (1999) Recommendation from the Scientific Committee on Occupational Exposure Limits for 2-Butanone. SCOEL/SUM/5.

US National Institute for Occupational Safety and Health (NIOSH) (1994) Immediately dangerous to life or health concentrations – 2-Butanone.