# Synthetic mineral fibres (SMF)

| CAS number: | — |
| --- | --- |
| Synonyms: | SMF, Man-made vitreous fibres (MMVF), synthetic vitreous fibres (SVF), Refractory Ceramic Fibres (RCF), glass fibres, glass wool, special-purpose glass fibres, ceramic dusts (aluminium silicate fibres) |
| Chemical formula: | — |
| Structural formula: | — |

Workplace exposure standard (retained)

| TWA: | 0.5 f/mL (respirable) 2 mg/m3 (inhalable dust) |
| --- | --- |
| STEL: | — |
| Peak limitation: | — |
| Notations: | — |
| IDLH: | — |
| Sampling and analysis: The recommended value is quantifiable through available sampling and analysis techniques. |  |

## Recommendation and basis for workplace exposure standard

A TWA of 0.5 f/mL (respirable) is recommended to protect for irritation of the respiratory tract and skin and to reduce the risk of lung fibrosis, pleural thickening, lung cancer and mesothelioma in exposed workers.

A complementary TWA of 2 mg/m3 for inhalable dust is recommended to minimise upper respiratory tract irritation from largely non-respirable fibres in exposed workers.

Given the limited data about carcinogenicity potential from the primary sources, it is recommended that a review of additional data sources is conducted at the next scheduled review.

## Discussion and conclusions

Synthetic mineral fibres (SMF) is a collective term used for amorphous vitreous fibres such as glass fibre, rock wool, slag wool and refractory ceramic fibres (RCF). Continuous glass filament is used as reinforcement in plastics and building products and in industrial fabrics. Glass wool and rock wool are used extensively in thermal and acoustic building insulation products. RCF are used primarily in industry as insulation for high-temperature applications such as furnaces, boilers and other heating equipment subjected to elevated temperatures.

The critical effects of exposure are transient, mechanical irritation of the respiratory tract and skin.

Exposure to SMF is reported to result in temporary irritation of the skin. Exposures to airborne SMF of concentrations greater than 1 f/mL are reported to cause temporary mechanical irritation of the upper respiratory tract especially when accompanied by high concentrations of non-fibrous particles.

The toxicity of RCF is considered to be higher than other SMF. RCF are more persistent. Size of RCF is mainly in the respirable range and it has potential to induce pleural thickening, lung cancer and mesothelioma.

Minimal adverse health effects reported in an extensive cohort study of exposures and health outcomes conducted in plants that manufactured RCF in Europe at an average exposure concentration of 1.06 f/mL.

RCF fibres caused lung fibrosis, pleural thickening, lung cancer and had the potential to cause mesothelioma in chronic inhalation studies in rats (ACGIH, 2018). RCF is classified as a Group 2B, possible carcinogenic to humans (IARC, 2002). Carcinogenicity of glass fibres is reported in animal studies by intrapleural and intraperitoneal injection and possibly intratracheal injection. However, these routes of exposure are considered irrelevant to workplace inhalational exposures based on the results of animal studies using glass wool, special-purpose glass fibres and rock/slag wool fibres. No pulmonary fibrosis or increase in lung cancer compared to controls in groups of rats exposed by inhalation to glass wool fibres at concentrations up to 222 f/mL. The evidence in humans does not support an increased risk of cancer for SMF other than RCF (ACGIH, 2018).

The available evidence indicates that a TWA of 0.5 f/mL (measured as respirable) is sufficient to protect for irritation of the respiratory tract and skin. The current TWA of 0.5 f/mL (respirable) is recommended to be retained. A secondary complementary TWA of 2 mg/m3 for inhalable dust is also recommended to be retained to minimise upper respiratory tract irritation from largely non-respirable fibres. No new data are available to suggest a change in the recommended TWA.

The data available in the sources is insufficient to derive a TWA based on carcinogenic effect for some fibres. It is recommended that a review of additional sources be conducted at the next scheduled review to investigate suitable data for this endpoint.

## Recommendation for notations

RCF and e-glass microfibres are classified as category 1B carcinogen according to the Globally Harmonized System of Classification and Labelling of Chemicals (GHS). Mineral wool is classified as category 2 carcinogen according to the GHS. Based on the available evidence, a review of this classification is recommended.

Not classified as a skin sensitiser or respiratory sensitiser according to the GHS.

There are insufficient data to recommend a skin notation.

# Appendix

### Primary sources with reports

| Source Year set Standard |  |
| --- | --- |
| SWA 2012 TWA: 0.5 f/mL (respirable), 2 mg/m3 (inhalable) |  |
| Revised in 2012. TWA for respirable fibres to be applied to synthetic mineral fibres; a collective term to describe synthetic fibres like glass wool, rock wool and refractory ceramic fibres. Previously, these fibres were known as SMF or man-made vitreous fibres (MMVF) (refer to exposure standard documentation for details). A complementary inhalable dust exposure standard of 2 mg/m3 (TWA) shall be applied to minimise mechanical irritation from largely non-respirable fibres. Inhalable dust exposure standard also applies to glass wool, rock (stone) wool, slag wool, continuous glass filament and low bio persistence MMVF and is not to take precedence over the respirable fibre standard, where the respirable fibre standard is applicable. |  |
| ACGIH 2001 TLV-TWA: Continuous Filament Glass Fibres: 1 f/mL (respirable), 5 mg/m3 (inhalable); Glass wool, rock wool, slag wool and special-purpose fibres: 1 f/mL (respirable); Refractory ceramic fibres (RCF): 0.2 f/mL (respirable) |  |
| Major categories of SMFs include continuous glass filaments, insulation wools (glass wool, rock wool, slag wool) and special-purpose fibres (e.g. glass fibres used in aerospace or filtration) and refractory ceramic fibres used for high-temperature insulation. Also labelled as synthetic vitreous fibres (SVF) or manmade vitreous fibres (MMVF). - Different forms of SMFs have different fibre sizes (nominal average diameter): - continuous glass filament: 3–25 µm (most fibres are non-respirable being 6 µm or higher in diameter) - insulation wool: 1–10 µm (nearly all wool products at 3 µm or higher) - special purpose glass fibre: 0.1–3 µm - RCF: 1.2–3 µm - Exposures to all SMF types may lead to transient mechanical irritation of the skin particularly to fibres >5 µm median aerodynamic diameter (MAD) and when accompanied by high concentrations of non-fibrous particles - Exposures to airborne SVF >1 f/mL cause temporary mechanical irritation of the URT, especially when accompanied by high concentrations of non-fibrous particles - RCF has higher potential to cause toxicity in humans compared with other SVF: - size of RCF largely in the respirable range - more durable or persistent than other SVF - Glass wool, special-purpose glass fibres and rock/slag wool fibres carcinogenic in test animals by intrapleural and intraperitoneal injection and possibly intratracheal injection. However, toxicity by these routes have no relevance to workplace exposure because: - animal inhalation studies of these fibres have not produced significant tumours - evidence in humans do not support or confirm an increased cancer risk - Animal inhalation studies of RCF suggest a risk of respiratory cancer. However, evidence in humans is insufficient to support or confirm an increased risk of cancer. Continuous filament glass fibres TLV-TWA of 1 f/mL for respirable fibre to minimise the potential for workers to experience the effect of mechanical irritation to the skin and mucous membranes. TLV-TWA of 5 mg/m3 for inhalable particulate fraction or non-respirable, continuous glass filament particles to minimise skin and mucous membrane irritation. Summary of data: Human data: - Most continuous glass filaments have a nominal MAD of 6–15 µm and are not considered respirable - Gravimetric methods of measurement should be used for continuous filament glass fibre products without respirable fibres because of absence of airborne, respirable fibres and in that case inhalable fibrous glass dust levels should not exceed 5 mg/m3 - May cause temporary irritation or itching of the skin in some workers due to mechanical effects of the fibres on the skin - Temporary irritation can occur in the nose, upper respiratory tract and eyes - Fibres >5 µm in diameter are most associated with mechanical irritation - Three cohort studies involving workers engaged in glass filament production found no indication in the results for those plants of significant adverse health effects - standardised mortality rates (SMR) indicated no excess lung cancer or non-malignant respiratory disease. Animal studies - No inhalation studies reported - No evidence of mesothelioma in rats following intrapleural and intraperitoneal injection of continuous glass filaments. Glass wool, rock wool, slag wool and special-purpose fibres TLV-TWA: 1 f/mL, respirable fibre to minimise skin and mucous membrane irritation. Summary of data: Human data glass wool - Exposure to glass fibres may produce a temporary mechanical irritation of the skin, nose and URT - Irritation noted when airborne concentrations exceeded 1 f/mL - Isolated reports of temporary irritation of the eye, especially in workers applying loose insulation - No serious or chronic problems associated with these complaints - Epidemiologic studies using a variety of experimental designs (primarily cohort and nested case-control designs) assessed the health effects of workers engaged in the production of glass wool fibres - Detailed synthesis of epidemiologic literature on risk of respiratory system cancer from glass fibres concluded: "taken together, the data indicate that among those occupationally exposed, glass fibres do not appear to increase the risk of respiratory system cancer." Animal data glass wool - No pulmonary fibrosis nor lung cancer reported in long-term inhalation studies of glass fibre in experimental animals have been reported, for example: - groups of 140 rats exposed by nose only inhalation 6 h/d, 5 d/wk at 3, 16 or 30 mg/m3 of glass wool fibre (27 f/mL, 142 f/mL or 222 f/mL, respectively) in a 2 yr study - no pulmonary fibrosis or increase in lung cancer found in the exposed animals compared to controls - Some intratracheal instillation studies with large doses produced pulmonary fibrosis. Human data rock wool and slag wool - Exposure to rock/slag wool fibres may produce a temporary mechanical irritation of the skin, nose and upper respiratory tract - Complaints of temporary eye irritation in workers applying loose insulation: - no serious or chronic problems associated with these complaints - A major U.S. cohort study of 1,846 workers engaged in the production of rock/slag wool: - statistically significant increased SMR of 135.6 for lung cancer cf local controls - no relationship with time from first exposure or with duration of exposure - in the nested case-control study of the rock wool/slag wool cohort, significant excess found for lung cancer mortality reported when controlling for smoking - a decreasing trend in lung cancer mortality with cumulative exposure to fibres <3 µm in diameter found among these workers - A study of 841 male slag wool production plant workers who worked for 1 yr or more in the plant: - 55 cases of lung cancer were compared to 98 matched controls - no increased risk of lung cancer associated with exposure - cigarette smoking found to be significant risk factor in this group due to extremely heavy cigarette smoking reported among the cases and overall high proportion of smokers in the entire exposed group - A major study including 10,115 workers in the production of rock/slag wool: - an SMR of 124 for lung cancer using death rates in local controls - increased lung cancer mortality in workers with ≥20 yr since first exposure; but considered no-significant compared to controls - statistically significant increase in lung cancer mortality (SMR of 223 cf death rates in local controls) in workers employed during early or intermediate year of production during which elevated dust levels were reported - restricting analysis to workers with at least 1 yr of employment considerably reduced the SMR. No further information provided; presumably removing initial one year of the production from the study results in reduced SMR (noting elevated dust levels in early phases of the production) - a separate review noted that some early contaminations with arsenic and other carcinogens may have been a factor in the increase of lung cancer in the early years of production - A survey of 185 workers engaged in the production of slag wool at two plants: - comparison of the prevalence of selected respiratory symptoms, smoking history, pulmonary function testing and chest X-ray - no significant excess of non-malignant respiratory disease - no adverse effects. Animal data rock wool and slag wool - No evidence of tumour in long-term inhalation studies on rats and hamsters. Human data special purpose glass fibres - An inhalation study of a special type of glass fibres (namely MMVF 33): - mesothelioma was induced in humans after 6 mo of exposure - full report not available at the time of ACGIH review. Animal data special purpose glass fibres - No animal data Refractory ceramic fibres (RCF) TLV-TWA: 0.2 f/mL for respirable fibre to minimise skin and mucous membrane irritation but consideration is given to fibre characteristics and the health outcomes: pleural thickening, lung cancer and mesothelioma potential. Summary of data: - Results of long-term inhalation studies in rats suggest RCF have potential to cause lung fibrosis, pleural thickening, lung cancer and mesothelioma - Epidemiological studies are less convincing; higher malignancies and fibrosis not expected possibly due to relatively brief exposures in the studies - Pleural thickening is reported in production workers exposed to RCF, possibly due to shorter latent period - RCF are more persistent than other SVF, size of RCF are mainly in the respirable range and toxicity is greater than other SMFs - Based on this evidence, a TLV-TWA lower than other SMFs is recommended. Human data refractory ceramic fibres - RCF may cause temporary irritation of the skin and URT among exposed workers - An association between exposure to RCF and pleural changes was reported in a study of 652 workers in several plants: - reported pleural plaques but no increase in interstitial fibrosis - A lung function study of 361 workers in RCF production between 1987 and 1994 (workers engaged in RCF production at time of study): - statistically significant decrease in pulmonary function (FVC) in workers involved in RCF production prior to 1980 - findings did not persist with analysis of follow-up production years - Evaluation of respiratory health of 742 workers at five RCF manufacturing plants in an industrywide pulmonary morbidity study: - respiratory symptoms were 2-5 times more prevalent in production workers compared to non-production workers - no significant changes in pulmonary function noted in non-smokers - predicted decrease in FVC and FEV1/10 yr of production work in current and past workers was of higher magnitude than that associated with past years of smoking - An extensive cohort study of exposures and health outcomes in all six plants in Europe that manufactured RCF: - analysis of respirable fibres and "total dust" of 464 shift personal samples covering 4 h or more; also collected 212 respirable dust and 57 inhalable dust samples: - overall mean exposure of 1.06 f/mL, varied between 0.68 and 3.18 f/mL depending on plant and time period - "total dust" concentrations ranged from 0.4–1.9 mg/m3 - very low respirable quartz concentrations with only 10/212 samples exceeding 0.005 mg/m3 - no asbestos in the airborne dust in fibre characterisation by scanning election microscope - evaluation of health effects by observing small opacities on X-ray films, the presence of rounded opacities, pleural plaques, pulmonary functions and prevalence of symptoms: - 695/775 workers (i.e. 90% workers) participated in the health survey; 79/211 former employees (37% workers) participated; average job tenure was 13 yr with 21% of the cohort having >20 yr of employment; 65% of men and 43% of women were current or former smokers - prevalence of ILO classification 0/0 was 49% in exposed and non-exposed workers; prevalence of 0/1 classification was higher in the exposed workers; prevalence of 1/0+ classifications was higher in the unexposed workers - pleural plaques observed in 5% of X-rays; odds ratios correlated with age but not with cumulative exposure estimates - Measurement of pulmonary function: - increased exposure caused decrease in FEV1 and FVC but only in smokers - symptoms survey: - 25/774 surveyed workers reported symptoms of chronic bronchitis - the odds of chronic bronchitis increased with recent exposure concentrations - similar observation for cough and phlegm but correlation not statistically significant - overall this cohort study suggests that RCF exposures results in minimal adverse health effects. Animal data refractory ceramic fibres - Inhalation of RCF reported to cause pulmonary fibrosis and increase in lung cancer in rats (no further information, presumably based on the findings listed below) - A separate study involving hamsters and rats: - no pulmonary changes - 1 hamster developed mesothelioma; no further information - 2 yr inhalation study in rats and hamsters, nose only exposure for 6 h/d, 5 d/wk at 30 mg/m3 (≡200 f/mL), rats exposed to 4 types of RCF (kaolin fibres, zirconia fibres, high-purity fibres, kaolin fibres after service), hamsters exposed to kaolin fibres: - fibrosis within 6 mo in hamsters and mesotheliomas in 42% hamsters by end of the study - biologically significant findings observed in rats: - developed fibrosis at 6–12 mo depending on type of RCF - significant increases in lung tumours by three types of RCF except kaolin fibres - mesotheliomas by three types of RCF although the number was low - no further information - Inhalation study in rats exposed at 3, 9 or 18 mg/m3 (25, 75 or 120 f/mL); fibres were 1 µm in diameter and 30 µm long: - pulmonary fibrosis at 18 mo of exposure in animals at 75 and 120 f/mL - no increase in pulmonary tumours - one mesothelioma at 75 f/mL - lung cancer through intratracheal instillation of RCF (no further information). |  |
| DFG 1997 NA |  |
| No MAK assigned due to carcinogenic potential. Review of various fibre types classified as ‘fibrous dust’ including ceramic fibres, glass fibres, man-made mineral fibres, rock wool and slag wool. Summary of additional data: - Fibrous dust consists of particles with a length to diameter ratio (L:D) exceeding 3:1 and which are >5 µm in length and <3 µm in diameter - The term "fibrous dust" applies to particles of these dimensions released from natural or synthetic materials or present in the product and can be emitted into the air. Ceramic dusts (renamed aluminium silicate fibres (RCF) - Animal studies by inhalation and IP administration indicate various types of ceramic fibres have carcinogenic potential, so the whole group of ceramic fibres considered to have carcinogenic potential - Classified as a Category 2 Carcinogen based unequivocally positive results (significant increase in tumour incidence) from studies with intraperitoneal, intrapleural or intratracheal administration. Glass fibres - Carcinogenic effect of glass fibres not clearly demonstrated to date from the inhalation studies - Glass fibres demonstrated to be carcinogenic in animal studies with intratracheal, intraperitoneal or intrapleural routes of administration - Glass fibres have potential for cell transforming and genotoxic activity - Considered clear evidence of carcinogenic potential of glass fibres from animal studies - Classified as a Category 2 Carcinogen based on unequivocally positive results (significant increase in tumour incidence) from studies with intraperitoneal, intrapleural or intratracheal administration. Rock wool - No increases in lung cancer risk detected in rock wool processing plant workers - Negative results in inhalation studies with rock wool: - some exposure may not have been sufficiently high to illicit carcinogenic effects - Studies with IP administration in animals used as evidence for carcinogenic potential of rock wool fibres - Classified as a Category 2 Carcinogen based on unequivocally positive results (significant increase in tumour incidence) from studies with intraperitoneal, intrapleural or intratracheal administration. Slag wool - Epidemiological studies in slag wool production plants: - an SMR of 148 for lung carcinoma for latency periods <20 yr - SMR of 191 for latency periods >20 yr - PAH with a high proportion of BaP also found at these workplaces - elevated risk of lung cancer cannot be unequivocally assigned to slag wool - Inhalation studies of exposed rats or golden hamsters (no further information): - tumours not detected - number of critical fibres in the aerosol very low, only low fibre concentrations detected in the lungs and the fibrosis very weak - The carcinogenic potential of slag wool cannot be assessed - Classified as a Category 3 Carcinogen: questionable results from inhalation studies or from studies with intraperitoneal, intrapleural or intratracheal administration. |  |
| SCOEL 2002 TWA: 10 f/mL |  |
| Review of man-made vitreous fibre (MMVF). Summary of additional data: - MMVF described as a generic name for an inorganic fibrous material manufactured primarily from glass, rock, minerals slag and processed inorganic oxides - The review uses test results of the combined fibres MMVF 10, MMVF 10a and MMVF 10.1 (all of which are derived from a typical type of building insulation fibreglass used in the USA) - Respirable fibres are considered those fibres with diameter of 5 µm (WHO definition) - No carcinogenic potential from up to MTD doses in long-term inhalation studies in rats and hamsters - The low pathogenicity of MMVF 10 correlates well with its low fibre persistence - TWA derivation based on repeated sub-chronic inhalation exposures in rats and hamsters (no further information): - a NOEL of 30x106 f/m3 (WHO defined respirable fibres) or 10x106 f with >20 µm based on BAL chemistry, BAL cells, lung cell proliferation and histopathology (no further information) - LOEL ranges between 150–200x106 f/mL or 50–80x106 f with >20 µm; 5-fold higher than NOEL; representing adaptive effects to fibre exposure in combination with an already significant impairment of particle clearance in rats - applies UF of 3 to the NOEL resulting in OEL of 10 f/mL or 10x106 f/m3 or 1 mg/m3 - no further information. |  |
| OARS/AIHA NA NA |  |
| No report. |  |
| HCOTN NA NA |  |
| No report. |  |

### Secondary source reports relied upon

| Source |  | Year | Additional information |
| --- | --- | --- | --- |
| AIOH |  | 2016 | - Synthetic mineral fibres (SMF), also known as man-made mineral fibres (MMMF), is a collective term used for amorphous vitreous fibres such as glass fibre, rock wool, slag wool and refractory ceramic fibres (RCF) - Evaluation considered older style SMF (types developed and used prior to the early 1990s) and newer developed SMF (types developed and in commercial use since the early 1990s), mainly the lower bio-persistence types - Recommended to limit exposure to 2.0 mg/m3 for inhalable, low-biopersistent SMF and to 0.5 f/mL for ‘odd’ forms of SMF such as RCF. |
| IARC |  | 2002 | - Evaluation for SMF resulted in the following findings based on the epidemiological and animal experimental studies (discussed above): - special-purpose glass fibres such as E-glass and ‘475’ glass fibres possibly carcinogenic to humans (Group 2B). - RCF possibly carcinogenic to humans (Group 2B). - insulation glass wool, continuous glass filament, rock (stone) wool and slag wool not classifiable as to their carcinogenicity to humans (Group 3). - considered not to evaluate newly developed fibres designed to be less bio-persistent (such as alkaline earth silicate or high-alumina and low silica wools) partly based on that human data were not available and due to difficulty in categorising these fibres into meaningful groups based on chemical composition. |

### Carcinogenicity — non-threshold based genotoxic carcinogens

| Is the chemical mutagenic? | Insufficient data |
| --- | --- |
| Is the chemical carcinogenic with a mutagenic mechanism of action? | Insufficient data |
| Insufficient data are available to determine if the chemical is a non-threshold based genotoxic carcinogen. |  |

## Notations

| Source | Notations |
| --- | --- |
| SWA | Carc. 1B, Carc. 2 |
| HCIS | NA |
| NICNAS | NA |
| EU Annex | NA |
| ECHA | NA |
| ACGIH | Continuous filament glass fibres: Carcinogenicity – A4 Glass wool, rock wool, slag wool and special-purpose fibres: Carcinogenicity – A3 RCF: Carcinogenicity – A2 |
| DFG | Ceramic Dust, Glass Fibres, Rockwool: Carcinogenicity – 2 Slag wool: Carcinogenicity – 3 |
| SCOEL | NA |
| HCOTN | NA |
| IARC | - Special-purpose glass fibres & RCF: Carcinogenicity – Group 2B - Insulation glass wool, continuous glass filament, rock (stone) wool and slag wool: Carcinogenicity – 3 |
| US NIOSH | NA |

NA = not applicable (a recommendation has not been made by this Agency); — = the Agency has assessed available data for this chemical but has not recommended any notations

### Skin notation assessment

Calculation

Insufficient data to assign a skin notation.

### IDLH

| Is there a suitable IDLH value available? | No |
| --- | --- |

## Additional information

| Molecular weight: | 206.18 |
| --- | --- |
| Conversion factors at 25°C and 101.3 kPa: | 1 ppm = NA; 1 mg/m3 = NA |
| This chemical is used as a pesticide: | ☐ |
| This chemical is a biological product: | ☐ |
| This chemical is a by-product of a process: | ☐ |
| A biological exposure index has been recommended by these agencies: | ☐ ACGIH ☐ DFG ☐ SCOEL |

## Workplace exposure standard history

| Year | Standard |
| --- | --- |
| Click here to enter year |  |

## References

American Conference of Industrial Hygienists (ACGIH®) (2018) TLVs® and BEIs® with 7th Edition Documentation, CD-ROM, Single User Version. Copyright 2018. Reprinted with permission. See the TLVs® and BEIs® Guidelines section on the ACGIH website.

Australian Institute of Occupational Hygienists (AIOH) Australian Institute of Occupational Hygienists (AIOH) (2016) Position paper: synthetic mineral Fibres (SMF) and occupational health issues.

Deutsche Forschungsgemeinschaft (DFG) (1997) Fibrous Dusts – MAK value documentation.

EU Scientific Committee on Occupational Exposure Limits (SCOEL) (2002) Recommendation from the Scientific Committee on Occupational Exposure Limits for MMVF 10. SCOEL/SUM/108.

International Agency for Research on Cancer (IARC) Man-made vitreous fibres. IARC Monographs – 81.