# Talc

| CAS number: | 14807-96-6 |
| --- | --- |
| Synonyms: | Steatite, talcum, French chalk |
| Chemical formula: | Mg3(OH)2Si4O10 |

Workplace exposure standard (amended)

| TWA: | 2 mg/m3 (respirable dust) |
| --- | --- |
| STEL: | — |
| Peak limitation: | — |
| Notations: | — |
| IDLH: | 1,000 mg/m3 |
| Sampling and analysis: The recommended value is quantifiable through available sampling and analysis techniques. |  |

## Recommendation and basis for workplace exposure standard

A TWA of 2 mg/m3 (respirable dust containing no asbestos and less than one per cent crystalline silica) is recommended to protect for pulmonary fibrosis and adverse changes in lung function in exposed workers.

## Discussion and conclusions

Talc is a mineral product with composition varying between geological deposits. The trade product may also contain various impurities. It is used extensively in a variety of industrial products (ceramics and pharmaceuticals) and cosmetics.

Critical effects of exposure are pulmonary fibrosis and changes in pulmonary (lung) function related to chronic inhalation exposures.

An increased prevalence of dyspnoea and lung opacities was identified among 166 workers with exposure at greater than 1.25 mg/m3 for 40 years (greater than 50 mg/m3-years) as respirable dust (ACGIH, 2018; DFG, 2006). A respiratory survey of 299 talc miners and millers from three separate locations with exposures ranging from 0.3 to 2.6 mg/m3 concluded that exposures less than or equal to an average of 1.1 mg/m3 (respirable particles) are not associated with pleural thickening (ACGIH, 2018).

Overload effects on the lungs at the lowest concentration administered of 2 mg/m3 reported in a four-week inhalation study in rats. Inflammation and fibrosis are reported in rats exposed at 6 and 18 mg/m3 (DFG, 2006). An increased incidence of adenoma and carcinoma of the lungs are observed in female rats and not in male rats at 18 mg/m3 in a chronic inhalation study. The tumours most likely manifested due to the general particle effect of granular biopersistent dusts (ACGIH, 2018; DFG, 2006). There is inadequate evidence in humans and limited evidence in animals for the carcinogenicity of inhaled talc that contains no asbestos, asbestiform fibres or crystalline silica (IARC, 2010; ACGIH, 2018)

The evidence indicates that adverse impacts on the pulmonary system are observed in humans due to exposure in the range of 2.5 mg/m3. Data from animal studies also suggest a retardation of the clearance of talc particles within this concentration range. Additional, adverse effects are not observed at exposures less than 1.1 mg/m3 in humans. Given this, the TWA of 2 mg/m3 (respirable dust) by ACGIH (2018) is recommended to protect for effects in the lungs. Given the critical effects occur at lower areas of the lungs, a TWA based on the respirable fraction is warranted.

## Recommendation for notations

Not classified as a carcinogen according to the Globally Harmonized System of Classification and Labelling of Chemicals (GHS).

Not classified as a skin sensitiser or respiratory sensitiser according to the GHS.

There are insufficient data to recommend a skin notation.

# Appendix

### Primary sources with reports

Source Year set Standard

SWA 1990 TWA: 2.5 mg/m3(inspirable dust)

The Exposure Standards Working Group recommended a TWA of 2.5 mg/m3 (inspirable dust) for talc containing <1% quartz and no detectable asbestos. This exposure standard is based on its historical use in Australia and should provide sufficient margin of safety in controlling pneumoconiosis and other respiratory effects. Note: Inspirable dust is now termed inhalable dust.

ACGIH 2010 TLV-TWA: 2 mg/m3, respirable particulate matter

TLV-TWA recommended to protect for pulmonary fibrosis and adverse effects to pulmonary function TLV applies only when talc contains no asbestos and < 1% crystalline silica Summary of data: Basis of TLV-TWA - No specific derivation provided - Exposed workers to respirable particles at an average of ≤1.1 mg/m3 is not associated with pleural thickening - No increases in prevalence of dyspnoea and lung opacities in workers exposed at 1.87 mg/m3 after adjustment for smoking and age: - in a follow-up study - no increased incidence in lung function impairment, small radiological opacities or respiratory symptoms exposed at an average of 1.46 mg/m3 Human data: - The adverse health outcomes associated with occupational exposure likely result from chronic exposures and display latency periods of several years: - evaluations of long-term workplace exposures reveal constant exposure concentrations over 20 yr and more prior to health outcomes - An increased prevalence of dyspnoea and lung opacities among workers with >50 mg/m3-yr, respirable dust exposure in a health study of 166 French mine and mill workers: - talc exposure characterised using 1,440 personal samples - geometric mean (range) estimated exposure at 1.87 mg/m3 (0.5–50 mg/m3), respirable dust; estimated cumulative exposure >50 mg/m3-yr for 95 subjects - after adjustment for smoking, both FVC and FEV1 decreased significantly with increasing exposure - prevalence of dyspnoea increased significantly after adjustment for smoking and age - prevalence of small radiological opacities was significantly related to age and to exposure after adjustment for age and smoking categories - No increased incidence in lung function impairment, small radiological opacities or respiratory symptoms during a 14.5 yr monitoring period at an average exposure concentration of 1.46 mg/m3 in a follow-up study of the same population (previous dot point) with additional workers from an Austrian mine: - talc deposits in these regions contain no asbestiform fibres - the silica content of the Austrian talc was <1% to 4%, and silica content in French site was undetectable at <3% - some concerns about the group size and methodology that make interpretations regarding negative outcomes difficult - Respiratory survey of 299 talc miners and millers from three U.S. states with mean exposures ranging 0.3–2.6 mg/m3 did not find an association with cumulative exposure but observed an increased prevalence of pleural thickening among long-term workers: - silica content in respirable dust was low; non-detectable in two states and 3.4% in one state - no asbestiform fibres present - prevalence of respiratory symptoms not associated with cumulative exposure or number of years worked - lung function was the same as control groups - prevalence of pleural thickening significantly associated with duration of employment (23% for >10 yr versus 2.5% for <10 yr) and not with cumulative exposure - pleural thickening correlated with smoking (no thickening in non-smokers), increased age and weight, years worked, reduced pulmonary function and higher average and cumulative talc exposures. Conclusion not talc related based on comparison of workers with and without pleural thickening - exposures to respirable particles at ≤1.1 mg/m3 (average) not associated with pleural thickening - Increased prevalence of cough, phlegm and wheezing with mean levels of exposure to respirable dust ranging from 0.47-3.55 mg/m3 reported in a study of 3 rubber tyre manufacturing workers: - pulmonary function tests, chest X-rays and respiratory questionnaires administered to 80 industrial grade talc-exposed workers and 189 non-exposed rubber workers - mean levels of talc exposure in exposed jobs ranged from 0.47-3.55 mg/m3, respirable dust, with most sampling averages < 1 mg/m3; exposure levels reported to have not changed substantially over the 20 yr prior to the study - exposed workers had statistically significantly greater prevalence of cough, phlegm and wheezing than control workers - talc workers with >10 yr of exposure had significantly decreased residual FEV1 - talc workers had clear increase in respiratory morbidity, despite absence of chest X-ray changes - Excesses of lung cancer in humans observed in several studies: - considered attributable to either asbestos or silica exposure. Animal data: - Pulmonary effects of talc baby powder in hamsters investigated: - groups of 100 animals exposed at 9.8 mg/m3 (MMAD = 4.9 µm), for a total of 1.5, 15, and 75 h over a 30-d period - 2 other groups of 100 hamsters exposed for 30 or 150 min/d for 300 d to 8.1 mg/m3 (MMAD = 6.0 µm) unless they died sooner - all animals survived - no lung tumours observed - Results of studies in rats and mice of both sexes exposed to aerosols containing 0, 6, or 18 mg/m3 talc for up to 113 wk (male rats), 122 wk (female rats) and 104 wk (mice) included: - increased incidence of benign and malignant pheochromocytomas of the adrenal gland seen in male rats at 18 mg/m3; NTP (1993) concluded "some evidence of carcinogenic activity" - increased incidence of alveolar/bronchiolar adenomas and carcinomas of the lung and benign and malignant pheochromocytomas of the adrenal glands in female rats at 18 mg/m3 warranted the NTP (1993) to conclude "clear evidence of carcinogenic activity" - exposure of mice associated with chronic, active inflammation and the accumulation of macrophages in the lung - hyperplasia of alveolar epithelium, squamous metaplasia or interstitial fibrosis not associated with inflammatory response in mice, reported contrast observations in the rat - similarly, incidences of pulmonary neoplasms in exposed and control groups of mice - based on these results, NTP concluded there was "no evidence of carcinogenic activity" of talc in male or female mice. Insufficient data to assign a skin or sensitiser notation or a TLV-STEL.

DFG 2006 Not assigned

Previous MAK of 2 mg/m3 withdrawn based on recent data from animal studies that indicate a retardation of clearance in this concentration range with changes in pulmonary function still observed in humans. Summary of additional data: - Recommends exposure should be below the general exposure standard for dust (respirable fraction) of 1.5 mg/m3. - Monitoring should also consider the fibre fraction contained in the material, especially industrial talc where high impurities occur - Refers to respiratory health study in French workers cited by ACGIH (2018): - pulmonary function changes after an exposure to >1.25 mg/m3 for 40 yr, i.e. at > 50 mg/m3-yr - Results of inhalation study in rats exposed for 4 wk (6 h/d, 5 d/wk) at 0, 2, 6, 18 mg/m3 included: - maximum pulmonary clearance ability for particles exceeded at 6 mg/m3 - overload effects on the lungs at the lowest concentration of 2 mg/m3 with lung burden increase - a concentration-related impairment of pulmonary function which increased in severity with an increasing exposure duration - inflammation and fibrosis were induced in male and female rats at 6 and 18 mg/m3 - refers to chronic inhalation study in male and female rats and mice cited by ACGIH (2018). - No adequate investigations available for plate-like crystals and fibres, which have high biopersistence, usually contained in talc. Suspected carcinogenicity could not be dispelled by available studies and therefore classified as Carcinogenicity category 3B.

SCOEL NA NA

No report.

OARS/AIHA NA NA

No report.

HCOTN NA NA

No report.

### Secondary source reports relied upon

| Source |  | Year | Additional information |
| --- | --- | --- | --- |
| IARC |  | 2010 | - Found insufficient or inconsistent evidence of increased risk for cancer in studies of workers occupationally exposed to talc - No data available on the genotoxic effects of exposure to talc in humans - Concluded inadequate evidence from epidemiological studies to assess whether inhaled talc not containing asbestos or asbestiform fibres causes cancer in humans - In the study of rats (cited by ACGIH), an excess incidence of malignant lung tumours seen in females. - An excess incidence of pheochromocytomas in the adrenal medulla in both sexes reported in the same study. Working Group was divided as to whether these rare tumours could be attributed to exposure to talc - Based on the one positive study, the Working Group found there was limited evidence of carcinogenicity of inhaled talc in experimental animals - Negative result reported in the limited number of studies available on the genetic toxicology of talc in vitro. |

### Carcinogenicity — non-threshold based genotoxic carcinogens

| Is the chemical mutagenic? | No |
| --- | --- |
| The chemical is not a non-threshold based genotoxic carcinogen. |  |

## Notations

| Source | Notations |
| --- | --- |
| SWA | NA |
| HCIS | NA |
| NICNAS | NA |
| EU Annex | NA |
| ECHA | NA |
| ACGIH | Carcinogenicity – A4 |
| DFG | Carcinogenicity – 3B |
| SCOEL | NA |
| HCOTN | — |
| IARC | Carcinogenicity – Group 3 |
| US NIOSH | NA |

NA = not applicable (a recommendation has not been made by this Agency); — = the Agency has assessed available data for this chemical but has not recommended any notations

### Skin notation assessment

Calculation

Insufficient data to assign a skin notation.

### IDLH

| Is there a suitable IDLH value available? | Yes |
| --- | --- |

## Additional information

| Molecular weight: | 379.3 |
| --- | --- |
| Conversion factors at 25°C and 101.3 kPa: | 1 ppm = 15.54 mg/m3; 1 mg/m3 = 0.06 ppm |
| This chemical is used as a pesticide: | ☐ |
| This chemical is a biological product: | ☐ |
| This chemical is a by-product of a process: | ☐ |
| A biological exposure index has been recommended by these agencies: | ☐ ACGIH ☐ DFG ☐ SCOEL |

## Workplace exposure standard history

| Year | Standard |
| --- | --- |
| Click here to enter year |  |

## References

American Conference of Industrial Hygienists (ACGIH®) (2018) TLVs® and BEIs® with 7th Edition Documentation, CD-ROM, Single User Version. Copyright 2018. Reprinted with permission. See the TLVs® and BEIs® Guidelines section on the ACGIH website.

Deutsche Forschungsgemeinschaft (DFG) (2006) Talc (without asbestos fibres) (respirable fraction) – MAK value documentation.

Health Council of the Netherlands (HCOTN) (2012) Talc. Evaluation of the carcinogenicity and genotoxicity. The Hague: Health Council of the Netherlands; publication no. 2012/11.

International Agency for Research on Cancer (IARC) (2010) Carbon Black, Titanium Dioxide, and Talc. IARC Monographs on the evaluation of the carcinogenic risk to humans.

US National Institute for Occupational Safety and Health (NIOSH) (1994) Immediately dangerous to life or health concentrations – talc.